SECTION 06 20 23

CASTWORKS™ INTERIOR MOLDED STONE (IMS) ARCHITECTURAL FORMS

Part 1 – GENERAL

1.1	RELATED DOCUMENTS

This specification is a general outline the CastWorks Interior Molded Stone (IMS) requirements, as they pertain to the overall project design. In all cases, the Manufacturer’s printed specifications shall govern the work of this section.

1.2	SUMMARY

- Section Includes:

  - Furnish all materials, labor, equipment, and services necessary for the supply and installation of Interior Molded Stone components as indicated on the drawings and contract documents, all in compliance with local codes and/or ordinances.

  - Work shall include supply of Interior Molded Stone components, installation, grouting (if required), patching, repairing and cleaning.

- Related Sections:

  - Section 05 10 00 Structural Metal Framing

  - Section 06 10 00 Rough Carpentry

  - Section 06 61 13 Simulated Stone Fabrications

  - Section 07 90 00 Joint Protection

  - Section 09 29 00 Gypsum Board

  - Section 09 75 13 Stone Wall Facing

  - Section 09 90 00 Painting and Coating

- Substitutions:

  - Manufacturers desiring to submit proposals other than CastWorks IMS shall, at least 10 days prior to the bid date, submit to the Architect all descriptive information of the system. These Manufacturers must have a minimum of five years’ experience with the system, provide photographs, and shop drawings of at least three projects similar in detail and scope with names, addresses and phone contacts of the respective Architects and installation contractors. Independent test data showing compliance with the specified system and three samples of similar details must also be submitted.

1.3	References

- American Society for Testing and Materials (ASTM):

  - ASTM C 947 (Modified) Standard Test Method for Flexural Properties of Thin-Section Glass-Fiber-Reinforced Concrete (Using Simple Beam With Third-Point Loading)

  - ASTM C 472 (Modified) Standard Test Methods for Physical Testing of Gypsum, Gypsum Plasters and Gypsum Concrete

  - ASTM D 638 (Modified) Standard Test Method for Tensile Properties of Plastics

  - ASTM D 256 (Method A) Standard Test Methods for Determining the Izod Pendulum Impact Resistance of Plastics

  - ASTM E136 Standard Test Method for Behavior of Materials in a Vertical Tube Furnace at 750°C

  - ASTM E 84 ASTM E84-18 Standard Test Method for Surface Burning Characteristics of Building Materials

1.4   SUBMITTALS

- Submit to the Architect a minimum of three 3" x 3" or 8" x 8" Interior Molded Stone flat samples of each color specified. These samples are for general appearance comparison only, as color variations will occur within and between components.

- Submit shop drawings for approval showing plans, sections, details, joint treatment, reinforcing, fastening methods and the relation of the Interior Molded Stone components to the surrounding construction.

- Product Data: Submit manufacturer’s technical data.

1.5	RESPONSIBILITY

- The Finish Carpentry Contractor shall install and tape the work under this section and he will be responsible for coordinating the installation with other trades.

1.6	MOCK-UP

- Prior to production, erect one proto-type on-site or at the Armstrong CastWorks plant, for review by the Architect. Once approved, the prototype will establish the standards by which the work will be judged.

1.7   DELIVERY, STORAGE, HANDLING AND PROTECTION

- Transport and handle Interior Molded Stone components in a manner that avoids excessive stresses or damage.

- Components displaying obvious damage must be rejected at site at time of delivery.

- Store the components in a controlled environment, weather protected, on level surfaces, with temporary supports as required. Do not stack or lean.

PART 2. – PRODUCTS

2.1	MANUFACTURER

- CastWorks by Armstrong World Industries, Inc.

2.2	MATERIALS

- Interior Molded Stone components shall be prefabricated with integrally pigmented high-density gypsum and aggregates free of resin and asbestos, reinforced with continuous filament mat or chopped strand fiber. Interior Molded Stone components shall be reinforced with steel or wood as required. Interior Molded Stone components are factory finished with a clear (non-gloss) sealer.

- Finishes: Colored and textured, in one of CastWorks IMS 12 standard colors. Custom color matching is available.

2.3		TOLERANCES (FABRICATION)

- Dimensional – all directions +/- 1/8"

- Thickness – skin +/- 1/8"

- Warpage or Bowing +/- 1/16"/foot

- Site conditions and normal manufacturing variations may require additional site work to maintain these tolerances.

2.4	PHYSICAL PROPERTIES

- Shell Thickness	1/4" - 3/8"

- Weight (depending on reinforcing)	2-1/2" - 5 lbs/sq.ft

- Density	95 lbs/cu.ft

- Flexural Strength (ASTM C-947-89 Modified)	1080 psi.

- Compressive Strength (ASTM C-472-90 Modified)	2840 psi.

- Tensile Strength (ASTM D-638-94b Modified)	155 psi.

- Impact Strength (ASTM D-256-93a, Method A)	.42 ft.lbs./inch of notch

- Fiber Content	4 - 6% by weight

- Flame Spread (ASTM E84-94) 0,	Class A

- Smoke Index (ASTM E84-94) 0,	Class A

  - INSPECTION

- The Architect or representative shall have access to the manufacturing facilities, either prior to contract award or thereafter, to inspect or verify compliance with the above specifications.

3.0	EXECUTION

3.1	PRE-INSTALLATION RESPONSIBILITY

- Field Measurements: Prior to manufacturing, the Installer will be responsible for obtaining all field dimensions for inclusion on the Manufacturers shop drawings.

- Coordination: The Installer will be responsible for the co-ordination of the installation with related sections, within the tolerances specified in the respective articles.

- Discrepancies: Prior to installation, the Installer shall check job site dimensions and conditions. Any discrepancies between design and field dimensions shall be brought to the attention of the General Contractor and the Architect.

- Material Quantities: The Installer will be responsible to order the correct material quantities, allowing for waste.

3.2	INSTALLATION

- Components shall be lifted with suitable devices.

- Components shall be installed plum and true. Shim where necessary.

- Pre-drill and countersink to 1/8" minimum below finished surface of unit.

- Fasten components with screws through face or back as indicated on
shop drawings.

- Adhere wall cladding (veneer) panels to substrate as indicated on shop drawings.

- Where components are suspended, use as a minimum 12 gauge steel wire and the suspension

- Allowable variation in material thickness at surface suspension points shall not exceed +/- 1/4".

- Framing, hangers etc., as specified elsewhere.

- Butt joints are to be caulked (as specified under Sealants and Caulking) or grouted. (Matching grout as furnished by CastWorks).

3.3	PATCHING AND CONTROL JOINTS

- Introduce control joints as required (35'-0" O.C.) under related section of Specification.

- Patch countersunk fasteners and any damage to match component texture, using materials furnished by CastWorks.

3.4 	CLEANING

- Clean Interior Molded Stone surface with a water/soap solution and rinse with a sponge. Excessive dirt can be removed with a multi-purpose spray cleaner such as ‘Spray Nine’. Remove and replace work that cannot be successfully cleaned or repaired.

3.5 	NOTES

- CastWorks IMS components shall be used for Interior Applications only.

- IMS may exhibit imperfections. To achieve satisfactory results, additional finishing may be required.

- IMS is made with natural, raw materials and may experience variation in color and texture.