SECTION 09 51 13

ACOUSTICAL PANEL CEILINGS

LYRA PB DIRECT APPLY CEILINGS AND WALLS

PART 1 – GENERAL

  - RELATED DOCUMENTS

Drawings and general conditions of Contract, including General and Supplementary Conditions and Divisions-1 Specification sections apply to work of this section.

  - SUMMARY

- Section Includes

  - Acoustical ceiling panels

  - Exposed grid suspension system

  - Wire hangers, fasteners, main runners, cross tees, and wall angle moldings

  - Perimeter Trim

- Related Sections

  - Section 09 50 00 - Ceilings

  - Section 09 51 14 – Acoustical Fabric Faced Panel Ceilings

  - Section 09 51 23 – Acoustical Tile Ceilings

  - Section 09 53 00 - Acoustical Ceiling Suspension Assemblies

  - Section 09 20 00 - Plaster and Gypsum Board

  - Section 01 81 13 - Sustainable Design Requirements

  - Section 01 81 19 - Indoor Air Quality Requirements

  - Section 02 42 00 - Removal and Salvage of Construction Materials

  - Division 23 - HVAC Air Distribution

  - Division 26 - Electrical

- ALTERNATES

  - Prior Approval: Unless otherwise provided for in the Contract documents, proposed product substitutions may be submitted no later than TEN (10) working days prior to the date established for receipt of bids.  Acceptability of a proposed substitution is contingent upon the Architect's review of the proposal for acceptability and approved products will be set forth by the Addenda.  If included in a Bid are substitute products that have not been pre-approved by the architect and included in the Addenda, the originally specified products shall be provided without additional compensation.

  - Submittals that do not provide adequate data for the product evaluation will not be considered.  The proposed substitution must meet all requirements of this section, including but not necessarily limited to, the following: Single source materials suppliers; Underwriters' Laboratories Classified Acoustical performance; Panel design, size, composition, color, and finish; Suspension system component profiles and sizes; Compliance with the referenced standards.

  - REFERENCES

- American Society for Testing and Materials (ASTM):

  - ASTM A 1008 Standard Specification for Steel, Sheet, Cold Rolled, Carbon, Structural, High-Strength Low-Alloy and High-Strength Low-Alloy with Improved Formability

  - ASTM A 641 Standard Specification for Zinc-Coated (Galvanized) Carbon Steel Wire

  - ASTM A 653 Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) by the Hot-Dip Process

  - ASTM C 423 Sound Absorption and Sound Absorption Coefficients by the Reverberation Room Method

  - ASTM C 635 Standard Specification for Metal Suspension Systems for Acoustical Tile and Lay-in Panel Ceilings

  - ASTM C 636 Recommended Practice for Installation of Metal Ceiling Suspension Systems for Acoustical Tile and Lay-in Panels

  - ASTM D 3273 Standard Test Method for Resistance to Growth of Mold on the Surface of Interior Coatings in an Environmental Chamber

  - ASTM E 84 Standard Test Method for Surface Burning Characteristics of Building Materials

  - ASTM E 580 Installation of Metal Suspension Systems in Areas Requiring Moderate Seismic Restraint

  - ASTM E 1111 Standard Test Method for Measuring the Interzone Attenuation of Ceilings Systems

  - ASTM E 1414 Standard Test Method for Airborne Sound Attenuation Between Rooms Sharing a Common Ceiling Plenum

  - ASTM E 1264 Classification for Acoustical Ceiling Products

- International Building Code

- ASHRAE Standard 62.1-2004, Ventilation for Acceptable Indoor Air Quality

- NFPA 70 National Electrical Code

- ASCE 7 American Society of Civil Engineers, Minimum Design Loads for Buildings and Other Structures

- International Code Council-Evaluation Services - AC 156 Acceptance Criteria for Seismic Qualification Testing of Non-structural Components

- International Code Council-Evaluation Services Report - Seismic Engineer Report

  - ESR 1308 - Armstrong Suspension Systems

- International Association of Plumbing and Mechanical Officials - Seismic Engineer Report

  - 0244 - Armstrong Single Span Suspension System

- California Department of Public Health CDPH/EHLB/Standard Method v1.2 2017

- LEED - Leadership in Energy and Environmental Design is a set of rating systems for the design, construction, operation, and maintenance of green buildings

- International Well Building Standard

- Mindful Materials

- Living Building Challenge

- U.S. Department of Agriculture BioPreferred program (USDA BioPreferred).

- Clean Rooms up to ISO Class 5 (Class 100)

  - SYSTEM DESCRIPTION

- Continuous/Wall-to-wall

  - SUBMITTALS

- Product Data: Submit manufacturer's technical data for each type of acoustical ceiling unit and suspension system required.

- Samples: Minimum 6-inch x 6-inch samples of specified acoustical panel; 8-inch-long samples of exposed wall molding and suspension system, including main runner and 4-foot cross tees.

- Shop Drawings: Layout and details of acoustical ceilings show locations of items that are to be coordinated with or supported by the ceilings.

- Acoustical Certifications: Manufacturer's certifications that products comply with specified requirements, including laboratory reports showing compliance with specified tests and standards.  For acoustical performance, each carton of material must carry an approved independent laboratory classification, such as Underwriter’s Laboratory (UL), of NRC, CAC, and AC.

  - If the material supplied by the acoustical subcontractor does not have an independent laboratory classification of acoustical performance on every carton, subcontractor shall be required to send material from every production run appearing on the job to an independent or NVLAP approved laboratory for testing, at the architect's or owner's discretion.  All products not conforming to manufacturer's current published values must be removed, disposed of, and replaced with complying product at the expense of the Contractor performing the work.

  - SUSTAINABLE MATERIALS

- Transparency: Manufacturers will be given preference when they provide documentation to support sustainable requirements for the following:  Material ingredient transparency, Removal of Red List Ingredients per LBCV3, Life Cycle impact information, Low-Emitting Materials, and Clean Air performance.

  - Health Product Declaration (HPD). The end use product has a published, complete Health Product Declaration with disclosure at a minimum of 1000ppm of known hazards in compliance with the Health Product Declaration Open Standard.

  - Declare Label.  The end use product has a published Declare label by the International Living Future Institute with disclosure of 100 ppm with a designation of Red List Free or Compliant (less than 1% proprietary ingredients).

  - Low Emitting products with VOC emissions data. Preference will be given to manufacturers that can provide emissions data showing their products meet any of the following: CDPH/EHLB/Standard Method v1.2-2017; Indoor Air Quality Certified to SCS-105 v4.2-2023

  - Life cycle analysis. Products that have communicated lifecycle data through Environmental Product Declarations (EPDs) will be preferred.

  - End of Life Programs/Recycling: Where applicable, manufacturers that provide the option for recycling of their products into new products at end-of-life through take-back programs will be preferred.

  - Products meeting LEED V4 requirements including:

    - Storage & Collection of Recyclables

    - Construction and Demolition Waste Management Planning

    - Building Life-Cycle Impact Reduction

    - Building Product Disclosure and Optimization Environmental Product Declarations

    - Building Product Disclosure and Optimization Sourcing of Raw Materials

    - Building Product Disclosure and Optimization Material Ingredients

    - Construction and Demolition Waste Management

  - QUALITY ASSURANCE

- Single-Source Responsibility: Provide acoustical panel units and grid components by a single manufacturer to ensure fit and function.

- Installer Qualifications:  Company specializing in performing specified work type, a minimum of three years of documented experience, and approved by the manufacturer.

- Fire Performance Characteristics: Identify acoustical ceiling components with appropriate markings of applicable testing and inspecting organization.

- Surface Burning Characteristics: Tested per ASTM E 84 and complying with ASTM E 1264 Classification.

  - DELIVERY, STORAGE, AND HANDLING

- Deliver acoustical ceiling units to project site in original, unopened packages and store them in a fully enclosed space where they will be protected against damage from moisture, direct sunlight, surface contamination, and other causes.

- Before installing acoustical ceiling units, permit them to reach room temperature and a stabilized moisture content.

- Handle acoustical ceiling units carefully to avoid chipping edges or damaging units in any way.

  - PROJECT CONDITIONS

- Space Enclosure:

  - HumiGuard Plus Ceilings: Building areas to receive ceilings shall be free of construction dust and debris. Products with HumiGuard Plus performance and hot dipped galvanized steel, aluminum or stainless-steel suspension systems can be installed up to 120°F (49°C) and in spaces before the building is enclosed, where HVAC systems are cycled or not operating. Cannot be used in exterior applications where standing water is present or where moisture will come in direct contact with the ceiling.

  - ALTERNATE CONSTRUCTION WASTE DISPOSAL

- Ceiling material being reclaimed must be kept dry and free from debris.

- Before disposing of ceilings, contact the Armstrong Recycling Center at 877-276-7876, select option #1 then #8 to review with a consultant the condition and location of building where the ceilings will be removed. The consultant will verify the condition of the material and that it meets the Armstrong requirements for recycling. The Armstrong consultant will help facilitate the process to recycle the ceiling.

- Recycling may qualify for LEED Credits:

  - LEED 2009 - Category 4: Material and Resources (MR)

    - Credit MRc2: Construction Waste Management

  - LEEDv4 - MRp2

    - Construction Waste Management Planning Qualifies as a material stream (non-structural) targeted for diversion. Ceilings will be source-separated and diverted through the Armstrong Ceiling Recycling Program.

  - LEEDv4-MRc5

    - Option 1: Divert ceilings to qualify for one of the 3 material streams (50%)

    - Option 2: Divert ceilings to qualify for one of the 4 material streams (75%)

  - WARRANTY

- Acoustical Panel: Submit a written warranty executed by the manufacturer, agreeing to repair or replace panels that fail within the warranty period.  Failures include, but are not limited to the following:

  - Acoustical Panels with HumiGuard® Max and HumiGuard® Plus performance:  sagging and warping

  - Acoustical panels with BioBlock® performance: growth of mold and mildew

  - Grid System:  rusting and manufacturer's defects

- Warranty Period:

  - Ceiling System: Thirty (30) years from date of substantial completion

- The Warranty shall not deprive the Owner of other rights the Owner may have under other provisions of the Contract Documents and will be in addition to and run concurrent with other warranties made by the Contractor under the requirements of the Contract Documents.

  - MAINTENANCE

- Extra Materials: Deliver extra materials to Owner.  Furnish extra materials described below that match products installed.  Packaged with protective covering for storage and identified with appropriate labels.

  - Acoustical Ceiling Units:  Furnish quality of full-size units equal to 5.0 percent of amount installed.

  - Exposed Suspension System Components:  Furnish quantity of each exposed suspension component equal to 2.0 percent of amount installed.

PART 2 – PRODUCTS

Attention Design Professional: Please edit Part 2 based on your project needs. Select product attributes and acceptable product item (s) that fit with the requirements of your project. Delete all items from the specification that do not relate to your project needs. Please refer to the Armstrong website for additional ceilings, suspension systems, perimeter trim options, and accessories. The related guide specifications for each of these items are available on the Armstrong website.

  - MANUFACTURERS

  - Ceiling Panels:

    - Armstrong World Industries, Inc.

  - Suspension Systems:

    - Armstrong World Industries, Inc.

2.2 ACOUSTICAL CEILING UNITS

  - Acoustical Panel Ceilings

    - Surface Texture:  Smooth Texture

    - Composition:  Fiberglass with Plant Based Binder

    - Color: White, Black, Concrete

    - Size:  24 in x 24 in, 24 in x 48 in, 30 in x 60 in, 30 in x 72 in, 48 in x 48 in

    - Edge Profile:  Square All-Sides

    - Noise Reduction Coefficient (NRC) ASTM C 423 Classified w/ UL label on product carton: 0.95,0.80

    - Flame Spread:  ASTM E 1264; Class A

    - Light Reflectance (LR) White Panel:  ASTM E 1477; 0.88

    - Dimensional Stability: HumiGuard Plus

    - Recycle Content: Up to 71% total recycled content. (Total recycled content: pre-consumer, post-consumer and post-industrial)

    - Material Ingredient Transparency: Health Product Declaration (HPD); Declare Label

    - Life Cycle Assessment: Third Party Certified Environmental Product Declaration (EPD)

    - Indoor Air Quality Certified to SCS-105 v4.2-2023

    - Basis of Design: LYRA PB Direct Apply, ITEM_______ as manufactured by Armstrong World Industries, Inc.

    - Substitutions: Refer to Alternates in Part 1.

  - Attachment Devices: Adhesive – TiteBond® GREENChoice Acoustical Ceiling Tile Adhesive

PART 3 – EXECUTION

3.1 EXAMINATION

- Do not proceed with installation until all wet work such as concrete, terrazzo, plastering and painting has been completed and thoroughly dried out, unless expressly permitted by manufacturer's printed recommendations.  (Exception: HumiGuard Max Ceilings)

3.2 PREPARATION

- Measure each ceiling area and establish layout of acoustical units to balance border widths at opposite edges of each ceiling.  Avoid use of less than half width units at borders and comply with reflected ceiling plans.  Coordinate panel layout with mechanical and electrical fixtures.

- Coordination: Furnish layouts for preset inserts, clips, and other ceiling anchors whose installation is specified in other sections.

  - Furnish concrete inserts and similar devices to other trades for installation well in advance of time needed for coordination of other work.

3.3 INSTALLATION

- Follow manufacturer installation instructions.

3.4 ADJUSTING AND CLEANING

- Replace damaged and broken panels.

- Comply with manufacturer's instructions for cleaning and touch up of minor finish damage.  Remove any ceiling products that cannot be successfully cleaned and or repaired. Replace with attic stock or new product to eliminate evidence of damage.