Architectural Components Group, Inc.

521 George St.

Marshfield, MO 65706

417-869-6777  /  417-869-7888 fax

info@acgiwood.com / acgiwood.com

ARCHITECTURAL SPECIFICATIONS FOR

ALLEGRO™ SERIES 2

CURVED CEILING PANELS

[Project Name]

[Project Location]

PART 1 – GENERAL

1.1 Description of Work:

- Work Included: Provide Architectural Components Group, Inc. Allegro™ AL2-xxxx-yyy curved ceiling panel system where shown on architectural drawings, and as specified in this specification.

1.2 Quality Assurance:

- Architectural Components Group, Inc. AL2-xxxx-yyy Allegro™ curved ceiling panel system for this project shall maintain the quality as instituted by the architect or A.W.I.

- Design Criteria: Manufacture of AL2-xxxx-yyy Allegro™ products shall be installed true and plumb to within manufacturing tolerance of 1/8” within 8’ of length.

- Product Construction: Wood shall be kiln dried to 10%. Cracking, checking and warpage of members will not be acceptable.

1.3 Submittals:

- Technical Data: Submit manufacturer’s data and installation instructions.

- Shop Drawings: Submit 4 copies of shop drawings showing all areas involved, attachment conditions and perimeter circumstances.

- Submittal Samples: Submit 1 product sample for approval by architect.

1.4 DELIVERY, STORAGE AND HANDLING

- Material MUST be STORED and installed only in secured ambient environment (humidity min. 35% -max. 55%, temperature not to exceed 86 deg.)

- Windows, doors and all wet-work must be completed before panels are removed from their packaging and installation begins. Handle carefully to avoid damaging.

- Panels should be stored and handled “flat,” e.g., not leaned against walls.

PART 2 – PRODUCTS

2.1 Acceptable Manufacturer:

Architectural Components Group, Inc., 521 George Street, Marshfield, MO 65706

Phone (417) 869-6777	Fax (417) 869-7888

2.2 Product Type

- Product Configuration: Allegro™ ceiling panel size shall be XX length by XX width size, with a X” reveal between panels. Panel edge shall be [matching veneer, painted, or black].

Nomenclature for AL2-4896-300:

AL2 = Allegro™ Series 2

4896 = a panel 48” wide by 96” length

300 = 3” tall perimeter trim

- The factory-attached perimeter trim shall be xx” tall. The perimeter trim shall be around each panel or around the ceiling cloud.

- Species: System shall consist of desired wood species veneer; laminate; or paint, e.g., cherry plain-sliced, book-matched.

- Finish: Wood veneer finish shall utilize ACGI clear finish with satin sheen or specify custom stain to match architect sample.

- Certification: Wood veneer shall or shall not be FSC certified. The core material shall or shall not be FSC certified. If FSC, Chain of Custody shall or shall not be provided.

- Substrate material shall or shall not be manufactured with no added urea formaldehyde.

- Suspension System: Allegro™ ceiling panels shall be suspended according to manufacturer’s suggested method of suspension. This system shall comply with Seismic Zone 2-4 regulations as required.

- Fire Rating: Allegro™ ceiling panels shall be constructed with components achieving a Class A (flame spread <25, smoke developed <450) rating.

PART 3 – EXECUTION

3.1 Installation

- Allegro™ ceiling panels shall be handled and installed with care in order to prevent surface and structure damage. Field cutting shall be kept to a minimum and performed as recommended by manufacturer.

- Allegro™ ceiling panels shall be suspended from the structure utilizing ACGI supplied suspension hangers from contractor supplied UniStrut.

PART – 4 WARRANTY

4.1 Product Warranty

- Architectural Components Group, Inc. shall warranty the Allegro™ ceiling panel system against manufacturer’s defects up to one year. Changes in finish or dimensions due to ultra violet light, excessive temperature or humidity conditions and/or abuse of any kind shall void any warranties from ACGI.

- Install conditions for the area should remain for the proper occupied conditions of humidity and temperature (humidity min. 25% - max. 55%, temperature not to exceed 86 degrees).

- The contractor shall warranty for one year all work from final acceptance of completed work.